Lab Summary 
note: Discard IGRP which is no longer used…

| vLab Title | 1137 Configuring RIP, EIGRP, and OSPF |
| --- | --- |
| Skills | Basic IP configuration of multiple routing protocols on a Cisco router which includes: Configuring IP RIP routing Configuring IP IGRP routing Configuring IP EIGRP routing Configuring OSPF routing Migrating from RIP to IGRP Migrating from IGRP to EIGRP Migrating from EIGRP to OSPF Verifying administrative distance Reviewing IP RIP debug information Reviewing IP IGRP transactions debug information Reviewing IP EIGRP debug information Reviewing OSPF adjacency debug information Interpreting the routing table Verifying network connectivity using ping |
| Level Of Difficulty | Basic |
| Course | ICND |
| Lab Length | 240 minutes |
| Certification | CCNA |
| Desired Learner Outcome | The user will build a network that utilizes distance vector (RIP and IGRP), hybrid (EIGRP), and link state (OSPF) routing protocols running in parallel to see how administrative distance, hop count, bandwidth, and load balancing play roles in what the router chooses as the best route. Configurations are then verified through show and debug commands. |
| Desired Network Outcome | A functioning three-router network using RIP, IGRP, EIGRP, and OSPF routing protocols. |
| Dependencies | The user is expected to have used the Cisco command line interface (CLI). |
| Author |  |
| Network Type | 3 Location Routed Network |
| Technology | Cisco |
| References | ICND 2.0 (Interconnecting Cisco Network Devices) - Module 5 Routing Protocols by Cisco Systems; Cisco IOS Command Documentation. Significant Commands Used in Lab : configure terminal enable end exit ping ip-address router igrp autonomous-system router rip router eigrp autonomous-system network network-number router ospf process-id network network-number wildcard-mask area-id show ip protocols show ip route debug ip rip debug ip igrp transactions debug ip eigrp debug ip ospf adj no debug all undebug all |

Launching Your Lab

The Status of your Lab is displayed at the top of the left navigation column.
Click the Start Lab Now button. A progress bar displays while the lab is being initialized. During this time you can view items under the Content area of the left navigation bar.

When Initialization is complete the Status changes to In Progress. The clock starts and a lab diagram displays in the main content area.
Click on the diagram that appears and your virtual lab experience will begin.

If your connection isn't working, verify connectivity by clicking Verify Connection in the Tools section on the left navigation bar.

Assignment

Configure RIP, IGRP, EIGRP, and OSPF on three routers over a serial network.

Story

The Challenge:

Your network has been configured to match the network diagram above, however the sites are complaining that there are some network segments which they can't reach. You have been given a maintenance window of 90 minutes to get full connectivity among the sites. You have decided to take advantage of the time to not only fix the problem, but to also test how running multiple routing protocols in parallel affect your routing tables.

Start out by testing connectivity from the Corporate router by pinging all remote interfaces and determine which are unreachable. For those networks that are unreachable, examine your routing table on the Corporate router to view known networks.

Enabling and Verifying RIP:

Configure RIP on all routers and then check the routing table on Corporate to see the new networks learned by RIP. Make note of the path(s) to the 150.10.3.0 network. RIP should see two equal cost paths. Verify that full connectivity has been established by pinging all remote interfaces. View the underlying exchanges of route information between RIP routers by issuing a RIP debug. After a few minutes, disable debugging.

Enabling and Verifying IGRP:

Migrate to IGRP on all routers without removing RIP. Choose any autonomous system number, but make sure it is the same for every router. Check the routing table on Corporate to see how the routing table has changed and make note of the path to the 150.10.3.0 network. The routing table should now show only one path. Verify the underlying exchanges of IGRP routing tables between routers using an IGRP debug. After a few minutes, disable debuggging.

Enabling and Verifying EIGRP:

Migrate to EIGRP on all routers using an autonomous system number of your choice, without removing IGRP. Check the routing table on Corporate to see how configuring EIGRP has affected it. Compare the administrative distances of the three routing protocols to see why only EIGRP routes are listed in the routing table.
EIGRP does not send regular periodic updates, but instead exchanges messages when a topology change occurs. Enable an EIGRP debug and then force a triggered EIGRP exchange by shutting down your Serial 0 interface and then re-enabling it.

Enabling and Verifying OSPF:

Configure all routers to be in a single OSPF area, Area 0. View the routing table on Corporate and you should find that nothing has changed. Compare the administrative distances of the four routing protocols to see why OSPF routes are not found in the routing table. Remove IGRP and EIGRP. Check the routing table again to see if OSPF routes are now visible. Because OSPF, like EIGRP, only sends updates when a change in the network has occured, enable an OSPF debug and then create a topology change by shutting down and re-enabling Serial 0. Verify that there is still full connectivity throughout the network by pinging all remote interfaces from Corporate.

Conditions

You are not to use static routes to fix any connectivity issues you may find in the lab.

Notes

* To be more effective in troubleshooting this lab, click on the Plan button found on the Locker page. * For a concise list of the tasks you are to perform in this lab, click on the Suggested Approach link on the Locker page. * If you get stuck, try the Sample Solution link on the Locker page.

Diagram

Suggested Approach 


The Suggested Approach identifies what to do based on the Scenario.
To find more details on how to do a specific task, access the Sample Solution.


PRIOR TO ENABLING ANY ROUTING PROTOCOLS

* Test connectivity from the Corporate router by pinging

all remote interfaces and determine which are unreachable.

* Examine the routing table on the Corporate router to view

known networks.

ENABLE AND VERIFY RIP

* Configure RIP on all routers.

* View how the routing table has changed on the Corporate router

and make note of the path to the 150.10.3.0 network.

* Verify that you can now ping all remote interfaces from Corporate.

* Verify the real-time exchange of RIP routes with a debug command.

ENABLE AND VERIFY IGRP

* Migrate to IGRP without removing RIP.

* View how the routing table has changed and make note of the path

to the 150.10.3.0 network.

* Verify the real-time exchange of IGRP routes with a debug command.

ENABLE AND VERIFY EIGRP

* Migrate to EIGRP without removing IGRP.

* View how the routing table has changed.

* Compare the administrative distances of the three routing protocols.

* Enable a debug to view EIGRP exchanges and force a topology change.

ENABLE AND VERIFY OSPF

* Configure all routers to be in a single OSPF area, area 0.

* View the routing table and you will find it unchanged.

* Compare the administrative distances of the four routing protocols

to see which is lowest.

* Remove IGRP and EIGRP.

* View how the routing table has changed.

* Enable a debug to view the formation of OSPF adjacencies and force

a topology change.

* Verify that you can still ping all remote interfaces from Corporate

Sample Solution

| Task Index |  |
| --- | --- |
| Task 1 | Test connectivity from the Corporate router by pinging all remote interfaces and determine which are unreachable. |
| Task 2 | Examine the routing table on the Corporate router to view known networks. |
| Task 3 | Configure RIP on all routers. |
| Task 4 | View how the routing table has changed on the Corporate router and make note of the path to the 150.10.3.0 network. |
| Task 5 | Verify that you can now ping all remote interfaces from Corporate. |
| Task 6 | Verify the real-time exchange of RIP routes with a debug command. |
| Task 7 | Migrate to IGRP without removing RIP. |
| Task 8 | View how the routing table has changed and make note of the path to the 150.10.3.0 network. |
| Task 9 | Verify the real-time exchange of IGRP routes with a debug command. |
| Task 10 | Migrate to EIGRP without removing IGRP. |
| Task 11 | View how the routing table has changed on the Corporate. |
| Task 12 | Compare the administrative distances of the three routing protocols. |
| Task 13 | Verify the real-time exchange of EIGRP messages with a debug command. |
| Task 14 | Configure all routers to be in a single OSPF area, Area 0. |
| Task 15 | View the routing table and you will find it unchanged. |
| Task 16 | Compare the administrative distances of the four routing protocols to see which is lowest. |
| Task 17 | Remove IGRP and and EIGRP. |
| Task 18 | View how the routing table has changed. |
| Task 19 | Enable a debug to view the OSPF adjacency changes. |
| Task 20 | Verify that you can still ping all remote interfaces from Corporate. |

Task 1

Test connectivity from the Corporate router by pinging all remote interfaces and determine which are unreachable.

Step 1 : Access the Corporate router and enter privileged EXEC mode. Action: {Click on the Corporate router found on the network diagram, then press [ENTER] to access the command line interface. enable Result: Configuring RIP, IGRP, EIGRP, and OSPF - Corporate Revision: Version 1.0 Date: March 12, 2003 Copyright 1998, 1999, Element K LLC All rights reserved User Access Verification Corporate>enable Corporate# Step 2 : From Corporate, determine which remote interfaces are unreachable. Action: Ping each interface on Richmond and Powhatan from the Corporate router. Result: Corporate#ping 150.10.1.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.1.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 28/31/32 ms Corporate#ping 150.10.5.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.5.2, timeout is 2 seconds: ..... Success rate is 0 percent (0/5) Corporate#ping 150.10.3.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.3.2, timeout is 2 seconds: ..... Success rate is 0 percent (0/5) Corporate#ping 150.10.2.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.2.3, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/35/36 ms Corporate#ping 150.10.3.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.3.3, timeout is 2 seconds: ..... Success rate is 0 percent (0/5) Corporate#ping 150.10.6.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.6.3, timeout is 2 seconds: ..... Success rate is 0 percent (0/5) Explanation: Routers can only reach networks that they have listed in their routing table. If a network is unknown, packets for that network are dropped. To see which networks the router is aware of, check the routing table.

Task 2

Examine the routing table on the Corporate router to view known networks.

Step 1 : Examine the routing table of Corporate. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 3 subnets C 150.10.4.0 is directly connected, Ethernet0 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 Explanation: Routers only know about directly connected networks unless you configure static routes or enable a dynamic routing protocol such as RIP, IGRP, EIGRP, or OSPF.

Task 3

Configure RIP on all routers.

Press [Enter] at any time to return to the device prompt. However, you do not need to wait for the device prompt to enter commands. Step 1 : Configure RIP to advertise the directly connected networks on Corporate. Action: router rip network 150.10.0.0 Result: Corporate#config t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#router rip Corporate(config-router)#network 150.10.0.0 Corporate(config-router)#end Corporate# Explanation: When advertising directly connected networks under the router rip process, the network statements are based on the class of the address. Since all three interfaces on Corporate (150.10.4.1/24, 150.10.1.1/24, and 150.10.2.1/24) belong to the same class B network, 150.10.0.0, you only need one network statement to enable RIP on all three interfaces. Step 2 : Configure RIP to advertise the directly connected networks on Richmond. Action: enable router rip network 150.10.0.0 Result: Richmond>en Richmond#conf t Enter configuration commands, one per line. End with CNTL/Z. Richmond(config)#router rip Richmond(config-router)#network 150.10.0.0 Richmond(config-router)#end Richmond# Step 3 : Configure RIP to advertise the directly connected networks on Powhatan. Action: enable router rip network 150.10.0.0 Result: Powhatan>en Powhatan#conf t Enter configuration commands, one per line. End with CNTL/Z. Powhatan(config)#router rip Powhatan(config-router)#network 150.10.0.0 Powhatan(config-router)#end Powhatan#

Task 4

View how the routing table has changed on the Corporate router and make note of the path to the 150.10.3.0 network.

Step 1 : Check the routing table on Corporate. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 6 subnets C 150.10.4.0 is directly connected, Ethernet0 R 150.10.5.0 [120/1] via 150.10.1.2, 00:00:24, Serial1 R 150.10.6.0 [120/1] via 150.10.2.3, 00:00:22, Serial0 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 R 150.10.3.0 [120/1] via 150.10.1.2, 00:00:24, Serial1 [120/1] via 150.10.2.3, 00:00:22, Serial0 Corporate# Explanation: The letter R indicates the routes dynamically learned by RIP. In the brackets following the RIP routes, you find [120/1]. The number 120 indicates the administrative distance for RIP and 1 refers to the hop count to reach a specific network. The routing table only installs the "best" route to reach a network. Notice that the path for the 150.10.3.0 network has two entries. This is because Corporate has two equal cost paths to reach it. In other words, it is one hop away via Richmond, but it is also one hop away via Powhatan. Since both are tied for the "best", they are both installed in the routing table. Packets destined for this network are load balanced over the two paths.

Task 5

Verify that you can now ping all remote interfaces from Corporate.

Step 1 : Verify connectivity to networks previously unreachable. Action: ping Result: Corporate#ping 150.10.1.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.1.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 28/31/32 ms Corporate#ping 150.10.5.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.5.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 28/31/32 ms Corporate#ping 150.10.3.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.3.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/42/68 ms Corporate#ping 150.10.2.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.2.3, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/34/36 ms Corporate#ping 150.10.3.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.3.3, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/41/60 ms Corporate#ping 150.10.6.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.6.3, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 36/36/36 ms Corporate# Explanation: Now that routing information is being exchanged throughout the network via RIP, full connectivity has been established.

Task 6

Verify the real-time exchange of RIP routes with a debug command.

Verify the network information associated with the entire router. Step 1 : Debug RIP updates on Corporate. Turn off debugging when finished. Action: debug ip rip u all Result: Corporate#debug ip rip RIP protocol debugging is on Corporate# 01:04:51: RIP: sending v1 update to 255.255.255.255 via Ethernet0 (150.10.4.1) 01:04:51: RIP: build update entries 01:04:51: subnet 150.10.1.0 metric 1 01:04:51: subnet 150.10.2.0 metric 1 01:04:51: subnet 150.10.3.0 metric 2 01:04:51: subnet 150.10.5.0 metric 2 01:04:51: subnet 150.10.6.0 metric 2 01:04:51: RIP: sending v1 update to 255.255.255.255 via Serial0 (150.10.2.1) 01:04:51: RIP: build update entries 01:04:51: subnet 150.10.1.0 metric 1 01:04:51: subnet 150.10.4.0 metric 1 01:04:51: subnet 150.10.5.0 metric 2 01:04:51: RIP: sending v1 update to 255.255.255.255 via Serial1 (150.10.1.1) 01:04:51: RIP: build update entries 01:04:51: subnet 150.10.2.0 metric 1 01:04:51: subnet 150.10.4.0 metric 1 01:04:51: subnet 150.10.6.0 metric 2 01:04:55: RIP: received v1 update from 150.10.1.2 on Serial1 01:04:55: 150.10.3.0 in 1 hops 01:04:55: 150.10.5.0 in 1 hops 01:04:55: 150.10.6.0 in 2 hops 01:05:00: RIP: received v1 update from 150.10.2.3 on Serial0 01:05:00: 150.10.3.0 in 1 hops 01:05:00: 150.10.5.0 in 2 hops 01:05:00: 150.10.6.0 in 1 hops Corporate#u all All possible debugging has been turned off Corporate# Explanation: Debugs show the underlying processes that are not normally seen on the console. Here we can see the entire routing tables being exchanged among the routers. The output indicates whether the information is sent or received, which interfaces are involved, and the metric (hop count) for each route. After you have viewed the exchanges, turn off all debugging by either using the no debug all command or enter u all which is short for undebug all.

Task 7

Migrate to IGRP without removing RIP.

Step 1 : Configure IGRP to advertise the directly connected networks on Corporate. Action: router igrp autonomous-system-number network 150.10.0.0 Result: Corporate#config t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#router igrp 100 Corporate(config-router)#network 150.10.0.0 Corporate(config-router)#end Corporate# Explanation: IGRP requires an autonomous system number. The number that you choose is arbitrary, but all IGRP routers in your network need to share the same AS number in order to share information. When advertising directly connected networks under the IGRP process, the network statements, like with RIP, are based on the class of the address. Since all three interfaces on Corporate (150.10.4.1/24, 150.10.1.1/24, and 150.10.2.1/24) belong to the same class B network, 150.10.0.0, you only need one network statement to enable IGRP on all three interfaces. Step 2 : Configure IGRP to advertise the directly connected networks on Richmond. Action: router igrp autonomous-system-number network 150.10.0.0 Result: Richmond#config t Enter configuration commands, one per line. End with CNTL/Z. Richmond(config)#router igrp 100 Richmond(config-router)#network 150.10.0.0 Richmond(config-router)#end Richmond# Step 3 : Configure IGRP to advertise the directly connected networks on Powhatan. Action: router igrp autonomous-system-number network 150.10.0.0 Result: Powhatan#conf t Enter configuration commands, one per line. End with CNTL/Z. Powhatan(config)#router igrp 100 Powhatan(config-router)#network 150.10.0.0 Powhatan(config-router)#end Powhatan#

Task 8

View how the routing table has changed and make note of the path to the 150.10.3.0 network.

Step 1 : Check the routing table on Corporate. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 6 subnets C 150.10.4.0 is directly connected, Ethernet0 I 150.10.5.0 [100/158350] via 150.10.1.2, 00:00:36, Serial1 I 150.10.6.0 [100/160350] via 150.10.1.2, 00:00:36, Serial1 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 I 150.10.3.0 [100/160250] via 150.10.1.2, 00:00:36, Serial1 Corporate# Explanation: The routes learned by RIP have now been replaced by IGRP routes, as indicated by the letter I in the table. Within the brackets following each network entry is the number 100, which is the administrative distance for IGRP, followed by a second number, which is the composite metric. With RIP, this metric was hop count. IGRP makes a calculation for its metric based on bandwidth and delay by default. Remember that the routing table only installs the "best" route to reach a network. With RIP, the path for the 150.10.3.0 network had two entries and was load balancing over the two. However, notice on the diagram that the connection from Corporate to Richmond is 64K, whereas the connection to Powhatan is 56K. IGRP takes bandwidth into consideration when choosing the best route and therefore only the route through Richmond for network 150.10.3.0 is placed in the table.

Task 9

Verify the real-time exchange of IGRP routes with a debug command.

Verify the connectivity between routers. Step 1 : Debug IGRP updates on Corporate. Turn off debugging when finished. Action: debug ip igrp transactions Result: Corporate#debug ip igrp transactions IGRP protocol debugging is on Corporate# 01:09:43: IGRP: received update from 150.10.2.3 on Serial0 01:09:43: subnet 150.10.4.0, metric 184671 (neighbor 160350) 01:09:43: subnet 150.10.5.0, metric 182671 (neighbor 158350) 01:09:43: subnet 150.10.6.0, metric 180671 (neighbor 1100) 01:09:43: subnet 150.10.1.0, metric 184571 (neighbor 160250) 01:09:43: subnet 150.10.3.0, metric 182571 (neighbor 158250) 01:10:18: IGRP: received update from 150.10.1.2 on Serial1 01:10:18: subnet 150.10.5.0, metric 158350 (neighbor 1100) 01:10:18: subnet 150.10.6.0, metric 160350 (neighbor 158350) 01:10:18: subnet 150.10.3.0, metric 160250 (neighbor 158250) 01:10:50: IGRP: sending update to 255.255.255.255 via Ethernet0 (150.10.4.1) 01:10:50: subnet 150.10.5.0, metric=158350 01:10:50: subnet 150.10.6.0, metric=160350 01:10:50: subnet 150.10.1.0, metric=158250 01:10:50: subnet 150.10.2.0, metric=180571 01:10:50: subnet 150.10.3.0, metric=160250 01:10:50: IGRP: sending update to 255.255.255.255 via Serial0 (150.10.2.1) 01:10:50: subnet 150.10.4.0, metric=1100 01:10:50: subnet 150.10.5.0, metric=158350 01:10:50: subnet 150.10.6.0, metric=160350 01:10:50: subnet 150.10.1.0, metric=158250 01:10:50: subnet 150.10.3.0, metric=160250 01:10:50: IGRP: sending update to 255.255.255.255 via Serial1 (150.10.1.1) 01:10:50: subnet 150.10.4.0, metric=1100 01:10:50: subnet 150.10.2.0, metric=180571 Corporate#u all All possible debugging has been turned off Corporate# Explanation: The debug shows the IGRP routes as they are being exchanged. The information includes whether the routing tables are sent or received, the IP address of the neighboring IGRP router, which interfaces are involved, and the metric.

Task 10

Migrate to EIGRP without removing IGRP.

Step 1 : Configure EIGRP to advertise the directly connected networks on Corporate. Action: router eigrp autonomous-system-number network 150.10.0.0 Result: Corporate#config t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#router eigrp 100 Corporate(config-router)#network 150.10.0.0 Corporate(config-router)#end Corporate# Explanation: Like IGRP, the EIGRP routing protocol uses an autonomous system number. This number needs to be the same for all EIGRP routers in your network. Network statements can be advertised using the major network number based on the class of the address, the same as it was configured for RIP and IGRP. However, EIGRP also can be configured to advertise networks with a mask different from the default. This is helpful in route summarization and in fixing problems with discontiguous networks. To advertise a different mask, a wildcard mask can be added to the EIGRP network statements. Step 2 : Configure EIGRP to advertise the directly connected networks on Richmond. Action: router eigrp autonomous-system-number network 150.10.0.0 Result: Richmond#config t Enter configuration commands, one per line. End with CNTL/Z. Richmond(config)#router eigrp 100 Richmond(config-router)#network 150.10.0.0 Richmond(config-router)#end Richmond# Step 3 : Configure EIGRP to advertise the directly connected networks on Powhatan. Action: router eigrp autonomous-system-number network 150.10.0.0 Result: Powhatan#config t Enter configuration commands, one per line. End with CNTL/Z. Powhatan(config)#router eigrp 100 Powhatan(config-router)#network 150.10.0.0 Powhatan(config-router)#end Powhatan#

Task 11

View how the routing table has changed on the Corporate.

Step 1 : Check the routing table on Corporate. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 6 subnets C 150.10.4.0 is directly connected, Ethernet0 D 150.10.5.0 [90/40537600] via 150.10.1.2, 00:03:00, Serial1 D 150.10.6.0 [90/41049600] via 150.10.1.2, 00:03:00, Serial1 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 D 150.10.3.0 [90/41024000] via 150.10.1.2, 00:03:00, Serial1 Corporate# Explanation: The routes learned by IGRP have now been replaced by EIGRP routes, as indicated by the letter D in the table. Within the brackets following each network entry is the number 90, which is the administrative distance for EIGRP, followed by a second number, which is the composite metric. EIGRP, like IGRP, uses bandwidth and delay by default to calculate its composite metric.

Task 12

Compare the administrative distances of the three routing protocols.

Step 1 : Enter a command on Corporate to verify the administrative distances of RIP, IGRP, and EIGRP. Action: show ip protocols Result: Corporate#show ip protocols Routing Protocol is "rip" Sending updates every 30 seconds, next due in 17 seconds Invalid after 180 seconds, hold down 180, flushed after 240 Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Redistributing: rip Default version control: send version 1, receive any version Interface Send Recv Triggered RIP Key-chain Ethernet0 1 1 2 Serial0 1 1 2 Serial1 1 1 2 Automatic network summarization is in effect Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 120 00:00:02 150.10.1.2 120 00:00:04 Distance: (default is 120) Routing Protocol is "igrp 100" Sending updates every 90 seconds, next due in 6 seconds Invalid after 270 seconds, hold down 280, flushed after 630 Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Default networks flagged in outgoing updates Default networks accepted from incoming updates IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0 IGRP maximum hopcount 100 IGRP maximum metric variance 1 Redistributing: igrp 100, eigrp 100 Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 100 00:00:57 150.10.1.2 100 00:00:29 Distance: (default is 100) Routing Protocol is "eigrp 100" Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Default networks flagged in outgoing updates Default networks accepted from incoming updates EIGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0 EIGRP maximum hopcount 100 EIGRP maximum metric variance 1 Redistributing: igrp 100, eigrp 100 Automatic network summarization is in effect Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 90 00:04:49 150.10.1.2 90 00:04:49 Distance: internal 90 external 170 Explanation: The show ip protocols command shows the details of any IP routing protocols that are enabled. It displays parameters, filters, and network information about the entire router. When using IGRP or EIGRP, check to make sure that the autonomous system number is correct (it must match on all neighboring routers, or else they will not pass routing information). Check the timers to make sure they match those of the adjacent routers. The last line of each routing protocol information block indicates the administrative distance. From the output we can see that RIP is 120, IGRP is 100, and EIGRP internal is 90. The source with the lowest administrative distance is chosen over the others when multiple sources are reporting information about the same network. Because of this, we only see EIGRP routes listed in the routing table.

Task 13

Verify the real-time exchange of EIGRP messages with a debug command.

Step 1 : Debug EIGRP message exchanges on Corporate. Turn off debugging when finished. Action: debug ip eigrp config t interface s0 shut no shut end u all Result: Corporate#debug ip eigrp IP-EIGRP Route Events debugging is on Corporate#config t Corporate(config)#int s0 Corporate(config-if)#shut Corporate(config-if)#no shut Corporate(config-if)# 01:19:12: IP-EIGRP: 150.10.1.0/24 - not in IP routing table 01:19:12: IP-EIGRP: Int 150.10.1.0/24 metric 4294967295 - 0 4294967295 01:19:12: IP-EIGRP: 150.10.5.0/24 - not in IP routing table 01:19:12: IP-EIGRP: Int 150.10.5.0/24 metric 4294967295 - 40000000 4294967295 01:19:13: IP-EIGRP: Processing incoming REPLY packet 01:19:13: IP-EIGRP: Int 150.10.1.0/24 M 47250176 - 45714176 1536000 SM 41024000 - 40000000 1024000 01:19:13: IP-EIGRP: Int 150.10.5.0/24 M 46763776 - 45714176 1049600 SM 40537600 - 40000000 537600 01:19:13: IP-EIGRP: Int 150.10.6.0/24 metric 46251776 - 45714176 537600 01:19:13: IP-EIGRP: Int 150.10.3.0/24 metric 46738176 - 45714176 1024000 01:19:13: IP-EIGRP: Int 150.10.1.0/24 metric 47250176 - 45714176 1536000 01:19:13: IP-EIGRP: Int 150.10.5.0/24 metric 46763776 - 45714176 1049600 01:19:14: IP-EIGRP: Int 150.10.1.0/24 metric 4294967295 - 45714176 4294967295 01:19:14: IP-EIGRP: 150.10.4.0/24 - do advertise out Serial1 01:19:14: IP-EIGRP: Int 150.10.4.0/24 metric 281600 - 256000 25600 01:19:14: IP-EIGRP: 150.10.2.0/24 - do advertise out Serial1 01:19:14: IP-EIGRP: Int 150.10.2.0/24 metric 46226176 - 45714176 512000 01:19:14: IP-EIGRP: 150.10.6.0/24 - do advertise out Serial1 01:19:14: IP-EIGRP: Int 150.10.6.0/24 metric 46251776 - 45714176 537600 01:19:14: IP-EIGRP: 150.10.3.0/24 - do advertise out Serial1 01:19:14: IP-EIGRP: Int 150.10.3.0/24 metric 46738176 - 45714176 1024000 01:19:14: IP-EIGRP: 150.10.5.0/24 - do advertise out Serial1 01:19:14: IP-EIGRP: Int 150.10.5.0/24 metric 46763776 - 45714176 1049600 Corporate(config-if)#end Corporate#u all All possible debugging has been turned off Corporate# Explanation: EIGRP, unlike RIP and IGRP, does not send periodic updates. Instead it sends information to other EIGRP routers only when changes occur. Because of this, after enabling debug ip eigrp, shut down the Serial 0 interface going to Powhatan, and then re-enable it. This creates a topology change in the network which forces the EIGRP messages to be triggered.

Task 14

Configure all routers to be in a single OSPF area, Area 0.

Step 1 : Configure OSPF to advertise the directly connected networks on Corporate to be in Area 0. Action: router ospf process-id network 150.10.0.0 0.0.255.255 area 0 Result: Corporate#conf t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#router ospf 1 Corporate(config-router)#network 150.10.0.0 0.0.255.255 area 0 Corporate(config-router)#end Corporate# Explanation: The number used in the router ospf command is not an autonomous system number. Instead, it is a process id. If you are only running one OSPF instance on a router (which is usually the case) it does not matter what number you choose as a process ID. The process ID also does not need to match other routers. It is only locally significant. To start OSPF on your interfaces, use a network statement followed by a wildcard mask. If you were to convert the wildcard mask to binary, each 0 would indicate an exact match bit. Each 1 would indicate a "don't care" bit. Using the wildcard mask 0.0.255.255 means that we want an exact match on the first two bytes, in this case, 150.10 and we don't care about the last two bytes. In other words, OSPF will be enabled on any interface matching 150.10.X.X. The area that the interfaces belong to is also specified. OSPF is capable of being broken into multiple smaller areas to improve on convergence time and to decrease the size of the routing tables. The scenario specified a single area for all interfaces, Area 0. Step 2 : Configure OSPF to advertise the directly connected networks on Richmond to be in Area 0. Action: router ospf process-id network 150.10.0.0 0.0.255.255 area 0 Result: Richmond#config t Enter configuration commands, one per line. End with CNTL/Z. Richmond(config)#router ospf 1 Richmond(config-router)#network 150.10.0.0 0.0.255.255 area 0 Richmond(config-router)#end Richmond# Step 3 : Configure OSPF to advertise the directly connected networks on Powhatan to be in Area 0. Action: router ospf process-id network 150.10.0.0 0.0.255.255 area 0 Result: Powhatan#config t Enter configuration commands, one per line. End with CNTL/Z. Powhatan(config)#router ospf 1 Powhatan(config-router)#network 150.10.0.0 0.0.255.255 area 0 Powhatan(config-router)#end Powhatan#

Task 15

View the routing table and you will find it unchanged.

Step 1 : Check the routing table on Corporate and make note of the networks dynamically learned. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 6 subnets C 150.10.4.0 is directly connected, Ethernet0 D 150.10.5.0 [90/40537600] via 150.10.1.2, 00:09:40, Serial1 D 150.10.6.0 [90/41049600] via 150.10.1.2, 00:09:40, Serial1 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 D 150.10.3.0 [90/41024000] via 150.10.1.2, 00:09:40, Serial1 Corporate# Explanation: The key above the routing table indicates that the letter D represents networks learned by EIGRP. To understand why OSPF routes are not shown, enter a command to compare the administrative distances of the routing protocols currently running.

Task 16

Compare the administrative distances of the four routing protocols to see which is lowest.

Step 1 : Enter a command on Corporate to verify the administrative distances of RIP, IGRP, EIGRP, and OSPF. Action: show ip protocols Result: Corporate#show ip protocols Routing Protocol is "rip" Sending updates every 30 seconds, next due in 20 seconds Invalid after 180 seconds, hold down 180, flushed after 240 Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Redistributing: rip Default version control: send version 1, receive any version Interface Send Recv Triggered RIP Key-chain Ethernet0 1 1 2 Serial0 1 1 2 Serial1 1 1 2 Automatic network summarization is in effect Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 120 00:00:19 150.10.1.2 120 00:00:23 Distance: (default is 120) Routing Protocol is "igrp 100" Sending updates every 90 seconds, next due in 10 seconds Invalid after 270 seconds, hold down 280, flushed after 630 Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Default networks flagged in outgoing updates Default networks accepted from incoming updates IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0 IGRP maximum hopcount 100 IGRP maximum metric variance 1 Redistributing: igrp 100, eigrp 100 Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 100 00:00:46 150.10.1.2 100 00:00:11 Distance: (default is 100) Routing Protocol is "eigrp 100" Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Default networks flagged in outgoing updates Default networks accepted from incoming updates EIGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0 EIGRP maximum hopcount 100 EIGRP maximum metric variance 1 Redistributing: igrp 100, eigrp 100 Automatic network summarization is in effect Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.2.3 90 00:10:11 150.10.1.2 90 00:10:11 Distance: internal 90 external 170 Routing Protocol is "ospf 1" Invalid after 0 seconds, hold down 0, flushed after 0 Outgoing update filter list for all interfaces is Incoming update filter list for all interfaces is Redistributing: ospf 1 Routing for Networks: 150.10.0.0 Routing Information Sources: Gateway Distance Last Update 150.10.6.3 110 00:01:31 150.10.5.2 110 00:01:31 Distance: (default is 110) Explanation: When remote networks are learned through multiple sources, there must be a way to determine which source is the most believable. The administrative distance provides this "believability" factor. The lower the number, the more believable the source. Comparing these four, EIGRP internal has the lowest with 90. Next is IGRP with an administrative distance of 100. OSPF routes are not found in the routing table since OSPF has an administrative distance of 110.

Task 17

Remove IGRP and and EIGRP.

Step 1 : Disable IGRP and EIGRP on Corporate. Action: no router igrp 100 no router eigrp 100 Result: Corporate#conf t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#no router igrp 100 Corporate(config)#no router eigrp 100 Corporate(config)#end Corporate# Explanation: Since OSPF will not be preferred when IGRP and EIGRP are also running, remove these two routing protocols so that OSPF routes will be listed in the routing table. Step 2 : Disable IGRP and EIGRP on Richmond. Action: no router igrp 100 no router eigrp 100 Result: Richmond#conf t Enter configuration commands, one per line. End with CNTL/Z. Richmond(config)#no router igrp 100 Richmond(config)#no router eigrp 100 Richmond(config)#end Richmond# Step 3 : Disable IGRP and EIGRP on Powhatan. Action: no router igrp 100 no router eigrp 100 Result: Powhatan#conf t Enter configuration commands, one per line. End with CNTL/Z. Powhatan(config)#no router igrp 100 Powhatan(config)#no router eigrp 100 Powhatan(config)#end Powhatan#

Task 18

View how the routing table has changed.

Step 1 : Check the routing table on Corporate. Action: show ip route Result: Corporate#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area * - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route Gateway of last resort is not set 150.10.0.0/24 is subnetted, 6 subnets C 150.10.4.0 is directly connected, Ethernet0 O 150.10.5.0 [110/1572] via 150.10.1.2, 00:01:59, Serial1 O 150.10.6.0 [110/1795] via 150.10.2.3, 00:01:59, Serial0 C 150.10.1.0 is directly connected, Serial1 C 150.10.2.0 is directly connected, Serial0 O 150.10.3.0 [110/3124] via 150.10.1.2, 00:01:59, Serial1 Corporate# Explanation: The only two routing protocols now running are RIP and OSPF. RIP has an administrative distance of 120, whereas OSPF is 110. Since the lower administrative distance is preferred, OSPF routes are now seen in the routing table.

Task 19

Enable a debug to view the OSPF adjacency changes.

Step 1 : Debug OSPF adjacency changes on Corporate. Turn off debugging when finished. Action: debug ip ospf adj config t interface s0 shut no shut end u all Result: Corporate#debug ip ospf adj OSPF adjacency events debugging is on Corporate#config t Enter configuration commands, one per line. End with CNTL/Z. Corporate(config)#int s0 Corporate(config-if)#shut Corporate(config-if)# 01:38:38: OSPF: Interface Serial0 going Down 01:38:38: OSPF: 150.10.4.1 address 150.10.2.1 on Serial0 is dead, state DOWN 01:38:38: OSPF: 150.10.6.3 address 150.10.2.3 on Serial0 is dead, state DOWN 01:38:38: %OSPF-5-ADJCHG: Process 1, Nbr 150.10.6.3 on Serial0 from FULL to DOWN, Neighbor Down 01:38:38: OSPF: Build router LSA for area 0, router ID 150.10.4.1, seq 0x8000000C 01:38:40: %LINK-5-CHANGED: Interface Serial0, changed state to administratively down 01:38:41: %LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0, changed state to down Corporate(config-if)# Corporate(config-if)#no shut 01:38:53: %LINK-3-UPDOWN: Interface Serial0, changed state to up 01:38:53: OSPF: Interface Serial0 going Up 01:38:53: OSPF: Build router LSA for area 0, router ID 150.10.4.1, seq 0x8000000D 01:38:54: %LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0, changed state to up 01:39:03: OSPF: Rcv DBD from 150.10.6.3 on Serial0 seq 0x610 opt 0x42 flag 0x7 len 32 mtu 1500 state INIT 01:39:03: OSPF: 2 Way Communication to 150.10.6.3 on Serial0, state 2WAY 01:39:03: OSPF: Send DBD to 150.10.6.3 on Serial0 seq 0x195E opt 0x42 flag 0x7 len 32 01:39:03: OSPF: NBR Negotiation Done. We are the SLAVE 01:39:03: OSPF: Send DBD to 150.10.6.3 on Serial0 seq 0x610 opt 0x42 flag 0x2 len 92 01:39:03: OSPF: Rcv DBD from 150.10.6.3 on Serial0 seq 0x611 opt 0x42 flag 0x3 len 92 mtu 1500 state EXCHANGE 01:39:03: OSPF: Send DBD to 150.10.6.3 on Serial0 seq 0x611 opt 0x42 flag 0x0 len 32 01:39:03: OSPF: Rcv DBD from 150.10.6.3 on Serial0 seq 0x612 opt 0x42 flag 0x1 len 32 mtu 1500 state EXCHANGE 01:39:03: OSPF: Exchange Done with 150.10.6.3 on Serial0 01:39:03: OSPF: Synchronized with 150.10.6.3 on Serial0, state FULL 01:39:03: %OSPF-5-ADJCHG: Process 1, Nbr 150.10.6.3 on Serial0 from LOADING to FULL, Loading Done 01:39:03: OSPF: Send DBD to 150.10.6.3 on Serial0 seq 0x612 opt 0x42 flag 0x0 len 32 01:39:03: OSPF: Build router LSA for area 0, router ID 150.10.4.1, seq 0x8000000E Corporate(config-if)# Corporate(config-if)#end Corporate#u all All possible debugging has been turned off Corporate# Explanation: OSPF, unlike RIP and IGRP, does not send periodic updates. OSPF is a link state routing protocol that sends information to other OSPF routers only when changes occur. OSPF routers will only send routing information to routers with which they have formed a formal relationship, called an adjacency. A number of conditions need to match in order to form an adjacency, including OSPF timers such as the hello and dead intervals. To see the adjacencies form along with the various states the routers go through in the process, shut down your Serial 0 interface and then re-enable it after enabling the debug ip ospf adj command. This creates a topology change and a breaking of the adjacency followed by its re-establishment.

Task 20

Verify that you can still ping all remote interfaces from Corporate.

Step 1 : Verify that there is full connectivity throughout the network. Action: ping Result: Corporate#ping 150.10.1.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.1.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/32/32 ms Corporate#ping 150.10.5.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.5.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/32/32 ms Corporate#ping 150.10.3.2 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.3.2, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 28/31/32 ms Corporate#ping 150.10.2.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 150.10.2.3, timeout is 2 seconds: !!!!! Success rate is 100 percent (5/5), round-trip min/avg/max = 32/35/36 ms

Check Results

After you have completed the scenario, you can compare your configurations with those provided here by issuing the show run command on each device.



Configuration Answer Key for Corporate

Corporate#show run

Current configuration:

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

service udp-small-servers

service tcp-small-servers

!

hostname Corporate

!

ip subnet-zero

!

cns event-service server

!

interface Ethernet0

description CONNECTION TO CORPORATE LAN

ip address 150.10.4.1 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial0

description CONNECTION TO POWHATAN - 56K

bandwidth 56

ip address 150.10.2.1 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial1

description CONNECTION TO RICHMOND - 64K

bandwidth 64

ip address 150.10.1.1 255.255.255.0

no ip route-cache

no ip mroute-cache

clockrate 64000

!

interface Serial2

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface Serial3

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface BRI0

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

router ospf 1

log-adjacency-changes

network 150.10.0.0 0.0.255.255 area 0

!

router rip

network 150.10.0.0

!

ip classless

no ip http server

!

!

banner motd ^C

Configuring RIP, IGRP, EIGRP, and OSPF - Corporate

Revision: Version 1.0

Date:  March 12, 2003

^C

!

line con 0

exec-timeout 0 0

transport input none

line aux 0

transport input all

line vty 0 4

no login

!

end

Configuration Answer Key for Richmond

Richmond#sho run

Current configuration:

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

service udp-small-servers

service tcp-small-servers

!

hostname Richmond

!

ip subnet-zero

!

cns event-service server

!

interface Ethernet0

description CONNECTION TO RICHMOND LAN

ip address 150.10.5.2 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial0

description CONNECTION TO CORPORATE - 64K

bandwidth 64

ip address 150.10.1.2 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial1

description CONNECTION TO POWHATAN - 64K

bandwidth 64

ip address 150.10.3.2 255.255.255.0

no ip route-cache

no ip mroute-cache

clockrate 64000

!

interface Serial2

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface Serial3

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface BRI0

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

!

router ospf 1

log-adjacency-changes

network 150.10.0.0 0.0.255.255 area 0

!

router rip

network 150.10.0.0

!

ip classless

no ip http server

!

!

banner motd ^C

Configuring RIP, IGRP, EIGRP, and OSPF - Richmond

Revision: Version 1.0

Date:  March 12, 2003

^C

!

line con 0

exec-timeout 0 0

transport input none

line aux 0

transport input all

line vty 0 4

no login

!

end

Configuration Answer Key for Powhatan

Powhatan#sho run

Current configuration:

!

version 12.1

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

service udp-small-servers

service tcp-small-servers

!

hostname Powhatan

!

ip subnet-zero

!

cns event-service server

!

interface Ethernet0

description CONNECTION TO POWHATAN LAN

ip address 150.10.6.3 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial0

description CONNECTION TO RICHMOND - 64K

bandwidth 64

ip address 150.10.3.3 255.255.255.0

no ip route-cache

no ip mroute-cache

!

interface Serial1

description CONNECTION TO CORPORATE - 56K

bandwidth 56

ip address 150.10.2.3 255.255.255.0

no ip route-cache

no ip mroute-cache

clockrate 56000

!

interface Serial2

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface Serial3

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

interface BRI0

no ip address

no ip route-cache

no ip mroute-cache

shutdown

!

router ospf 1

log-adjacency-changes

network 150.10.0.0 0.0.255.255 area 0

!

router rip

network 150.10.0.0

!

ip classless

no ip http server

!

!

banner motd ^C

Configuring RIP, IGRP, EIGRP, and OSPF - Richmond

Revision: Version 1.0

Date:  March 12, 2003

^C

!

line con 0

exec-timeout 0 0

transport input none

line aux 0

transport input all

line vty 0 4

no login

!

end